# Video Subsystem

## Text mode

### 120 columns

### 67 rows (half row thrown out)

### 8040 chars, 16080 bytes

### Cell value : 16 bit width

#### Color : 8 bits

##### Foreground 3 bits

##### Background 3 bits

##### Pallet Index 2 bits

#### Character : 8 bits

##### Look up from character map

### Character Map: 16x16x256

#### 256 cells

#### Bit mask 0: background, 1 foreground

#### 15

### Palette Definition

#### 4 palettes

#### 8 values per palette

#### 16 bits wide

##### ??NN RRRR GGGG BBBB

##### 12LSB are RGB color

##### 2 ISB are Blinking Pattern

##### 2MSB invert control

###### 00 => Mate, not inverted

###### 01 => Mate, inverted

###### 11 => Self inverted

###### 10 => undefined

### Blinking Patterns

#### 4 configurable patterns